Technical Specifications

This project implements a configurable dual-clock asynchronous FIFO designed for reliable data transfer between two independent and asynchronous clock domains. The FIFO is parameterizable in terms of data width and FIFO depth, allowing it to be adapted for different applications. The design uses separate write (wr_clk) and read (rd_clk) clock domains with no fixed phase or frequency relationship between them. Binary read and write pointers are maintained locally and converted into Gray-code pointers before crossing clock domains, reducing the possibility of CDC errors because only one Gray-code bit changes during each pointer increment. The synchronized pointers are transferred using at least two-stage flip-flop synchronizers to provide metastability protection. The FIFO generates glitch-free FULL and EMPTY status flags in their respective clock domains and also supports programmable ALMOST-FULL (AFULL) and ALMOST-EMPTY (AEMPTY) watermark thresholds. The design includes asynchronous reset assertion with safe reset de-assertion in each clock domain, preventing recovery and removal timing violations. Verification includes a self-checking testbench, different clock frequencies and phase relationships, integer and non-integer clock ratios, simultaneous read/write operations, burst transfers, FIFO boundary conditions, reset testing, CDC verification, and MTBF-oriented stress testing. The design is intended for a complete RTL-to-GDSII ASIC flow, including synthesizable SystemVerilog RTL, CDC analysis, SDC timing constraints, synthesis, static timing analysis, area analysis, and waveform-based verification. The final implementation should demonstrate no data loss, no data duplication, correct FIFO ordering, reliable CDC operation, glitch-free status flags, and proper timing closure after synthesis.